Pointwise (1×1) Convolution on the AMD XDNA2 NPU — `conv2d_1x1_opt`

This is the optimized pointwise-convolution operator. It starts from the plain GEMM mapping (a 1×1 conv is a matrix multiply) and adds one hardware-aware optimization — the cols→M axis swap — that turns the array's biggest weakness on real CNN layers (column starvation) into throughput. This document is the full picture: what the operator computes, how the work is spread across the NPU, the optimization, and the measured results.

**If you remember one thing:** a 1×1 convolution is a matrix multiply, and the

only question that matters for speed is *how you map its three dimensions onto a

4-row × 8-column grid of cores*. Picking that mapping per layer is the whole game.

1. What a 1×1 convolution is (and why it is a GEMM)

A feature map is a stack of channels: every pixel holds a vector of C_in numbers. A 1×1 (pointwise) convolution looks at one pixel at a time and mixes its channels into C_out new ones with a weight table that is the same for every pixel:

output_channels = Weights · input_channels          (Weights is C_out × C_in)

Line up all the pixels and this is exactly a matrix multiply. With batch = 1:

| GEMM role | tensor | torch.nn layout | flattened | frame |
| --- | --- | --- | --- | --- |
| A [M, K] | weight w | [C_out, C_in, 1, 1] | [C_out, C_in] | row-major |
| B [K, N] | activation x | [1, C_in, H, W] | [C_in, H·W] | row-major |
| C [M, N] | output y | [1, C_out, H, W] | [C_out, H·W] | row-major |

So the conv maps to GEMM as M = C_out, K = C_in, N = H·W (pixels), and C = A × B. Crucially, the flattened torch.nn tensors are already the row-major GEMM buffers — weights→A, activation→B, output→C — so in the plain mapping there are no transposes and no host reshaping, and the result comes back in NCHW. (Trying to "fix" a layout with a DMA transpose is illegal anyway: a strided bf16 transpose reads 2-byte elements that are not 4-byte aligned, which the NPU DMA rejects. The escape from that trap for the swap is §4.)

Scope: the direct [K, N] mapping treats one image at a time. Non-tile-aligned shapes are handled by padding (§3); the axis swap and multi-column execution are implemented (§4–§5); `batch > 1` is supported by running the per-image program once per image with shared weights (§7).

2. The NPU and how a GEMM is spread across it

NPU2 (AMD XDNA2, Strix) is a grid of tiles: 4 compute rows × 8 compute columns = up to 32 cores, a row of mem tiles (L2 scratchpad, one per column), and a row of shim tiles (the door to DRAM). Data moves L3 (DRAM) → L2 (mem) → L1 (core-local) by DMA engines, streamed through ObjectFifos (conveyor belts with a fixed number of slots). Cores compute only on L1 data.

The output C (M×N) is cut into m × n tiles and mapped onto the grid:

•  Columns ↔ pixels (N). Each column owns an n-wide slice of pixels.

•  Rows ↔ output channels (M). Each of the 4 rows owns an m-tall slice of C_out.

•  K = C_in is the reduction, chopped into k-blocks; each core accumulates its

tile over K/k multiply-adds (C_tile += A_tile × B_tile).

A (weights) is broadcast across columns and distributed across rows; B (activation) is broadcast across rows and distributed across columns. When there are more output tiles than cores, each core processes several tiles in sequence (n_c_col_tiles_per_core = N/(n·cols), n_c_row_tiles_per_core = M/(m·4)).

3. Padding (host-side, layout-preserving)

The array wants each mapped dim to tile evenly: M a multiple of m·4, K of k, N of n·cols. Real conv shapes rarely comply (C_out = 40, H·W = 49, …). op.py rounds M/K/N up and the pad_weight / pad_activation / pad_output helpers zero-extend the flattened tensors in their natural storage order; unpad_output slices the logical region back. Zero-padded channels/pixels contribute exactly 0, so the padded result equals the logical result zero-extended, and the design and kernel never see a ragged shape. This mirrors GEMM's own pad_A/pad_B (same principle, same layout-aware, never a host transpose).

op.py exposes pad_overhead (padded MACs ÷ useful MACs) so tests report how much array time is spent multiplying zeros.

4. The optimization: the cols→M axis swap

The problem — column starvation

The array is 4 rows × 8 columns. The plain mapping hard-wires columns ↔ pixels. For tall layers (many output channels, few pixels — the late stages of every ResNet/ConvNeXt/EfficientNet) the 8-wide column axis is spent on a handful of pixels (mostly zero-padding), while the large C_out dimension — where the work is — gets only 4-way row parallelism. Across 8 real models, 34% of all 1×1-conv layers are tall, and every 7×7 (N=49) layer actually slows down as columns are added: the columns are pure padding.

The idea

Map columns ↔ C_out (8-wide, where the channels are) and rows ↔ pixels (4-wide, plenty for a small pixel count). Put the wide axis where the work is.

The escape — feed everything as-stored, transpose in the kernel

Swapping forces the activation into the A (rows) operand as [pixels, C_in] — but it is stored [C_in, pixels], i.e. column-major, and a bf16 DMA transpose is illegal. The fix mirrors exactly how GEMM already handles b_col_maj/c_col_maj: feed each operand as-stored and let the compute kernel transpose the small `r×s` sub-blocks in-register (aie::transpose). No host transpose anywhere.

| design dim | = conv dim | operand (as fed) | flag |
| --- | --- | --- | --- |
| M̂ (rows) | pixels H·W | A = activation [C_in, pixels] | a_col_maj |
| K̂ | C_in | (reduction) | — |
| N̂ (cols) | C_out | B = weights [C_out, C_in] | b_col_maj |
| output | — | C = output [C_out, pixels] (NCHW) | c_col_maj |

a_col_maj is the one new piece; it is the exact mirror of b_col_maj (col-major a_dims, a col-major A_tiles DRAM tiler, and an in-kernel aie::transpose), gated by -DA_COL_MAJ so the baseline path is untouched. It requires ≥ 4 columns (so each shim feeds exactly one A row-tile); the swap is a multi-column optimization, so this is not a real restriction. All three buffers keep their natural torch storage — only the operand slot order and the col-major flags change — so pad_*/unpad_output are mapping-independent.

When it fires — the auto heuristic

The swap only pays off when the plain mapping actually starves the columns — otherwise its extra in-kernel transposes cost more than the occupancy they buy. swap_mn=None (default) swaps iff

ceil(pixels / tile_n) · 2 ≤ num_columns    AND    ceil(C_out / tile_n) > ceil(pixels / tile_n)

i.e. the plain mapping leaves at least half the columns idle and C_out fills more of them. Force it with swap_mn=True/False.

5. Results (NPU2, cols=8, tile 16/64/64, bf16 in / f32 accumulate)

Protocol: 10 warmup rounds, then median over 50 timed on-device samples. Throughput is on useful conv MACs (2·C_out·C_in·H·W, padding excluded), so it is directly comparable across mappings.

Correctness

All 22/22 real-model shapes (ResNet-50, ResNeXt-50, MobileNetV2/V3, EfficientNet-B0, ConvNeXt-T, DenseNet-121, YOLOv5l) pass the torch golden reference (rel/abs tol 5e-3) at cols=8 — tall shapes take the swap, wide shapes stay plain.

Swap speedup across the real-model suite (opt ÷ baseline)

| C_in / C_out / N | speedup |  | C_in / C_out / N | speedup |
| --- | --- | --- | --- | --- |
| 768 / 3072 / 49 | 6.27× |  | 1152 / 192 / 49 | 1.44× |
| 1024 / 2048 / 49 | 4.58× |  | 112 / 672 / 196 | 1.28× |
| 512 / 2048 / 49 | 4.40× |  | 80 / 480 / 196 | 1.22× |
| 2048 / 512 / 49 | 3.28× |  | 1024 / 512 / 196 | 1.19× |
| 3072 / 768 / 49 | 3.23× |  | 96 / 576 / 196 | 1.12× |
| 384 / 1536 / 196 | 1.84× |  | median (11 swapped) | 1.84× |

The 7×7 (N=49) tail — the layers plain GEMM handles worst — gains 2.9–6.3×. Wide layers keep the plain mapping (auto-heuristic declines), so the operator never trades throughput away; the one tall layer that already fills its columns (1024/512/400, N=400) is correctly left on the plain mapping.

Column scaling — why the swap exists

Throughput (GF/s) baseline / opt as columns are added:

| C_in/C_out/N | cols=1 | cols=2 | cols=4 | cols=8 |
| --- | --- | --- | --- | --- |
| 768/3072/49 | 211/213 | 206/207 | 187/650 | 165/1074 |
| 1024/2048/49 | 218/218 | 217/214 | 204/633 | 188/807 |
| 384/1536/196 | 224/222 | 382/390 | 549/567 | 488/915 |

For the 7×7 layers the baseline is flat or *falling as columns are added (768/3072/49: 211 → 165 GF/s) — extra columns are pure padding. The swap makes throughput climb* (213 → 1074). It engages precisely when a column count starves the layer (14×14 layers wait until cols=8).

Scalar vs vectorized kernel (cols=8)

| C_in / C_out / N | scalar GF/s | vectorized GF/s | vec / scalar |
| --- | --- | --- | --- |
| 768 / 3072 / 49 | 6.7 | 1042 | 156× |
| 384 / 1536 / 196 | 6.7 | 853 | 128× |
| 2048 / 512 / 49 | 6.6 | 641 | 97× |
| 64 / 256 / 3136 | 7.5 | 711 | 95× |

The scalar kernel is pinned at ~6.6 GF/s (one MAC at a time); the vectorized aie::mmul kernel reaches 575–1042 GF/s — median 97×, up to 156×. Stacked, for 768/3072/49: scalar 6.7 → vectorized-plain 165 (25×) → vectorized+swap 1042 (156×).

6. The two compute kernels

Selected by use_scalar:

•  Vectorized (default): uses the hardware aie::mmul instruction on pre-shuffled

r×s/s×t sub-blocks (the layout the DMA delivers). Hundreds of GF/s. Also holds the a/b/c_col_maj in-register transposes the swap relies on.

•  Scalar (reference): a plain triple loop on plain row-major tiles — slow but

easy to trust, used as a correctness oracle and performance baseline. It accumulates in 32-bit float, so keep prio_accuracy=True for it.

design.py streams plain tiles to the scalar kernel and shuffled tiles to the vectorized kernel, so each gets the layout it expects.

7. Using the operator

op = Conv2d1x1(

batch=1, H=7, W=7, C_in=1024, C_out=2048,   # conv shape (torch.nn dims)

tile_m=16, tile_k=64, tile_n=64,            # GEMM tile

num_aie_columns=8,                          # sweep this yourself

use_scalar=False, prio_accuracy=True,

swap_mn=None,                               # None=auto, True/False=force

context=ctx,

)

rows, cols = op.input_operands(w, x)   # ordered for the mapping (handles the swap)

inputs  = {"A": rows, "B": cols}

outputs = {"C": op.pad_output(y).flatten()}

•  `num_aie_columns` is the main throughput lever — sweep it ({1,2,4,8}); the

swap engages automatically at ≥ 4 columns when a layer is starved.

•  `tile_m/k/n` is the tile-shape lever (16/64/64 is the proven default; it is

autotuned per shape — see below).

•  `swap_mn` forces or disables the axis swap; input_operands returns the two

input buffers already in the design's arg order for the chosen mapping.

Tile autotuning. The tile shape is tuned empirically, not analytically — an analytical tile model was unreliable here, and some tiles regress catastrophically (e.g. 32/64/64 → 36 GF/s on 2048/512/49 vs 513 for the default) in ways no static model predicts. autotune.py sweeps a small candidate set on-device per (shape, cols), verifies correctness, and caches the fastest in tile_cache.json; get_optimal_tile() reads it and CONV_TUNE=1 runs the suite on the tuned tiles. The cache ships tuned for the full 22-shape suite at cols=8 (any other (shape, cols) falls back to the 16/64/64 default). Gains are modest but real — up to 1.41×, median ~1.11× over 16/64/64 — and the best tile varies per shape (no single tile wins: 16/128/64 helps tall big-K layers, larger 32/64/64/64/64/64 win on aligned mid-size, 16/64/64 stays best for several). Selection lives in the host (like softmax's get_optimal_*), so op.py is unchanged.

Complementary to the swap. Tuning helps least exactly where the swap helps most, for a mechanical reason: on a tall 7×7 layer the swap moves the ~49 pixels onto the row axis, which pins m = 16 (a bigger row tile would be pure padding) — every N=49 layer keeps `m=16` in the cache. The tuner's real gains (up to 1.41×) land on the 14×14 / wide layers, where the pixel axis is large enough that a bigger tile (m=32–64) is free and amortizes per-tile overhead — often the wide layers the swap declines. Occupancy (columns) and tile amortization are orthogonal axes of the same problem, so the two levers cover disjoint parts of the workload.

CONV_COLS=8 python autotune.py             # tune the shape set -> tile_cache.json

CONV_COLS=8 CONV_TUNE=1 pytest … test.py   # run the suite on tuned tiles

Batching (`batch > 1`). A batch is batch independent 1×1 convs that share the same weights (Y_b = W · X_b). Because the batch axis is stored outside the channels ([batch, C_in, H·W]), a batch is not one big GEMM — it is one dispatch per image of this same per-image program, with the weights reused. op.py accepts batch > 1 and exposes image_operands(w, x), which yields the (rows, cols) buffers for each image; the caller runs one dispatch per image and concatenates:

op = Conv2d1x1(batch=4, H=7, W=7, C_in=1024, C_out=2048, …)

for b, (rows, cols) in enumerate(op.image_operands(w, x)):      # x: [4, C_in, H, W]

op_func({"A": rows, "B": cols}, out=op.pad_output(y[b]).flatten())

This is mapping-agnostic — it works identically for the plain and swapped mappings. (A single-dispatch batched design, gemv/transpose-style, is possible but interacts with the swap's operand flip; per-image dispatch reuses the design untouched.)

The test.py suite draws its shapes from 8 real models and sweeps columns via the CONV_COLS env var (CONV_COLS=8 pytest … test.py); test_conv2d_1x1_batched covers batch > 1 on both a wide (plain) and a tall (swapped) layer.

8. Where each piece lives

| File | Runs on | Responsibility |
| --- | --- | --- |
| op.py | Host | Operator definition: conv→GEMM mapping, the cols→M swap decision + auto-heuristic, layout-preserving padding, per-image batching (image_operands), kernel/compile flags (-DA_COL_MAJ etc.), build-artifact naming. |
| design.py | Host | The shared whole-array GEMM design; adds the a_col_maj col-major A path (mirror of b_col_maj) and the scalar-kernel tiling. Generates the MLIR. |
| aie_kernels/aie2p/conv2d_1x1.cc | NPU core | Vectorized aie::mmul kernel (with a/b/c_col_maj in-register transposes) and the scalar oracle. |
| reference.py | Host | Golden answer: builds NCHW x / OIHW w, computes y = F.conv2d(x, w). |
| test.py | Host | Real-model suite; feeds flattened torch tensors via input_operands, checks against the reference, reports throughput + pad_overhead. Holds the tile-selection helper get_optimal_tile(). |
| autotune.py | Host | Empirical tile-size sweep; writes the fastest tile per (shape, cols) to tile_cache.json. |

9. Gotchas

•  Build cache & flags. use_scalar / prio_accuracy / emulate_bf16… /

a/b/c_col_maj all change the generated program and are folded into the operator name and kernel-object name, so each combination is its own cache entry. If you suspect a stale build: rm build/Conv2d1x1_* build/conv1x1_*.

•  The scalar path needs `prio_accuracy=True` to stay within tolerance.

•  Never transpose on the host. The swap feeds col-major operands as-stored and

transposes in-kernel; a strided bf16 DMA transpose is illegal (not 4-byte aligned). This is the same rule GEMM follows for b/c_col_maj.

•  The swap needs ≥ 4 columns. At 1–2 columns a tall layer auto-falls-back to the

plain mapping (the A row-distribution only lines up with ≥ 4 columns).

•  `batch > 1` is one dispatch per image (shared weights), not a single fused

GEMM — the per-image N is H·W, never batch·H·W. Iterate with op.image_operands(w, x).

•  A 1×1 conv adds no *compute* over a GEMM. All conv-specific logic is the

dimension mapping and the swap decision in op.py; the compute kernel is the shared GEMM one.